# Packet Tracer - Back Up Configuration Files (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Objectives

Part 1: Establish Connectivity to TFTP Server

Part 2: Transfer the Configuration File from TFTP Server

Part 3: Backup Configuration and IOS to TFTP Server

# Background / Scenario

In this activity you will restore a configuration from a backup and then perform a new backup. Due to an equipment failure, a new router has been put in place. Fortunately, backup configuration files have been saved to a Trivial File Transfer Protocol (TFTP) Server. You are required to restore the files from the TFTP Server to get the router back online as quickly as possible.

# Instructions

## Establish Connectivity to the TFTP Server

Note: Because this is a new router, the initial configuration will be performed using a console connection to the router.

    - Click PCA, then the Desktop tab, followed by Terminal to access the RTA command line.

    - Configure and activate the Gigabit Ethernet 0/0 interface. The IP address should match the default gateway for the TFTP Server.

Router(config)# interface g0/0

Router(config-if)# ip address 172.16.1.1 255.255.255.0

Router(config-if)# no shut

    - Test connectivity to TFTP Server. Troubleshoot, if necessary.

## Transfer the Configuration File from the TFTP Server

    - From privileged EXEC mode, issue the following command:

Router# copy tftp running-config

Address or name of remote host []? 172.16.1.2

Source filename []? RTA-confg

Destination filename [running-config]? <cr>

The router should return the following:

Accessing tftp://172.16.1.2/RTA-confg...

Loading RTA-confg from 172.16.1.2: !

[OK - 785 bytes]

785 bytes copied in 0.001 secs

RTA#

%SYS-5-CONFIG_I: Configured from console by console

RTA#

    - Issue the command to display the current configuration.

### Question:

What changes were made?

ไฟล์การตั้งค่า (Configuration) ที่เก็บสำรองไว้บน TFTP Server ได้ถูกโหลดเข้าสู่หน่วยความจำของเร้าเตอร์เรียบร้อยแล้ว และชื่อโฮสต์ (Hostname) ของเร้าเตอร์ได้ถูกเปลี่ยนเป็น RTA (สังเกตได้จากหน้าจอคำสั่งที่เปลี่ยนจาก Router# เป็น RTA#)

The configuration stored on the TFTP Server was loaded into the router and the hostname of the router changed to RTA.

    - Issue the appropriate show command to display the interface status.

### Question:

Are all interfaces active?

ไม่ อินเทอร์เฟซ G0/1 ยังคงถูกปิดการใช้งานอยู่ (ขึ้นสถานะว่า administratively down)

No, G0/1 is administratively down.

    - Correct any issues related to interface problems and test connectivity between PCA and the TFTP server.

## Back Up Configuration and IOS to TFTP Server

    - Change the hostname of RTA to RTA-1.

    - Save the configuration to NVRAM.

    - Copy the configuration to the TFTP Server using the copy command:

RTA-1# copy running-config tftp:

Address or name of remote host []? 172.16.1.2

Destination filename [RTA-1-confg]? <cr>

    - Issue the command to display the files in flash.

    - Backup the IOS in flash to the TFTP Server using the following command:

RTA-1# copy flash tftp:

Source filename []? c1900-universalk9-mz.SPA.151-4.M4.bin

Address or name of remote host []? 172.16.1.2

Destination filename [c1900-universalk9-mz.SPA.151-4.M4.bin]? <cr>

### Question:

What special character repeatedly displays indicating that the IOS file is being copied to the TFTP server successfully?

เครื่องหมายตกใจ หรือ อัศเจรีย์

The exclamation point !.

    - Open the TFTP Server and click the Services tab, select TFTP, and scroll through the list of IOS files.

### Question:

Has the IOS file c1900-universalk9-mz.SPA.151-4.M4.bin been copied to the TFTP Server?

ใช่ ไฟล์ IOS ดังกล่าวได้ถูกคัดลอกไปเก็บไว้บน TFTP Server เรียบร้อยแล้ว (คุณสามารถยืนยันได้ด้วยตัวเอง โดยการคลิกที่เครื่อง TFTP Server > เลือกแท็บ Services > เลือกเมนู TFTP ซ้ายมือ แล้วเลื่อนหาชื่อไฟล์นี้ในรายการ)

Yes, the c1900-universalk9-mz.SPA.151-4.M4.bin file is listed in the files on the TFP server.

End of document